| LIFT & SHIFT CHECKLIST BY SITE TEAM This checklist is to be completed by the site project team prior to acceptance of delivery of the electrical equipment to the project site. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item for Delivery: |  |  |  |  |  |
| Item Description: |  |  |  |  |  |
| Aspect | Checklist |  | Yes | No | N/A |
| Legislation | - Safety, Health and Welfare at Work Act 2005 duties met - Compliance with GA Regulations 2007 (Work Equipment, Lifting Equipment, Manual Handling) - Construction Regulations 2013 applied where relevant |  | ☐ | ☐ | ☐ |
| Planning | - Risk assessment completed - Method statement prepared - Site survey conducted - Ground conditions verified |  | ☐ | ☐ | ☐ |
| Roles | - Appointed Person / Supervisor identified - Competent operators and riggers assigned - Contractor compliance confirmed |  | ☐ | ☐ | ☐ |
| Equipment | - Lifting equipment has valid thorough examination certification(s) - Skoots/jacks/winches inspected and fit for use - Lifting accessories marked with WLL |  | ☐ | ☐ | ☐ |
| Safe System of Work | - Lifting/moving plan documented - Rigging/ de-rigging per manufacturer’s guidance - Load stability ensured - Communication system in place |  | ☐ | ☐ | ☐ |
|  | - Use skid-mounted assemblies to minimise handling |  | ☐ | ☐ | ☐ |
|  | - Identify Centre of Gravity (CoG) and design top cable entry accordingly |  | ☐ | ☐ | ☐ |
|  | - Ensure certified lifting equipment is aligned with the CoG for safe handling |  | ☐ | ☐ | ☐ |
|  | - Provide clear Risk Assessments Method Statements (RAMS) |  | ☐ | ☐ | ☐ |
|  | - Locate electrical equipment near exterior entrances in greenfield projects |  | ☐ | ☐ | ☐ |
|  | - Verify floor loadings, ceiling heights, and access routes to accommodate the equipment |  | ☐ | ☐ | ☐ |
|  | - Fit lifting eyes or approved lifting points wherever possible |  | ☐ | ☐ | ☐ |
|  | - Ensure floors are level and stable |  | ☐ | ☐ | ☐ |
|  | - Design plinths for forklift compatibility |  | ☐ | ☐ | ☐ |
|  | - Heavy components should be positioned at the base of the assembly where possible (and for LV, compliance with IEC EN 61439-1 temperature rise test verifications). |  | ☐ | ☐ | ☐ |
|  | - Always follow manufacturer’s lifting instructions |  | ☐ | ☐ | ☐ |
| Training | - Operators hold CSCS or equivalent - Competent Supervisors and Appointed Persons (for lifting operations) - Training records up to date |  | ☐ | ☐ | ☐ |
| Emergency Preparedness | - Rescue plan documented - First aid provision confirmed - Emergency contacts briefed |  | ☐ | ☐ | ☐ |
| Additional Comments: |  |  |  |  |  |

This template is referenced as Appendix 4 in the CIF Guidance on the Lift and Shift of Electrical Equipment. Available at: https://cif.ie/