# TOOLBOX TALK – ‘Ladders & Safe Working at Height’

Contributor: Michael Murphy, Laois Scaffolding Hire Ltd

“Steps, or step ladders are one of the most used, and abused, pieces of equipment on a construction site. When abused and misused, they have enormous potential to cause accidents and injuries. Falling from a height is a major cause of fatalities in the construction industry. More than half of falls from a height of over 2 metres result in death or a life changing injury. Any fall from a height has the potential to cause serious life changing injuries. Most falls that result in serious injuries are from a relatively short distance 2-3 metres. Fall prevention is much better than dealing with the consequences of a fall from height. Remember it’s not the fall, it’s the sudden stop that causes the damage”. (Michael Murphy, Head of Health and Safety, Laois Scaffolding Hire Ltd.)

Contributor’s Shared Advice:

- Plan and organise the work schedule - Risk Assess all work at height, before attempting to start work - Select the most appropriate access equipment - Ladders are essentially a means of access/egress and should only be used as working platforms for very short duration tasks, where alternative platforms would be impracticable, and where such tasks can be carried out safely using a ladder - Only professional ladders should be used, - All ladders must be certified and should have a record of inspection - All ladders should be in good condition - Ladders must be suitable for the work being undertaken and suitably secured (preferably tied off at the top using both stiles to prevent both sideways slip and rotation) - All operatives must be aware of the hazards and risks involved in the work they are undertaking - Training in working at height equipment must be provided - Always carry out visual check of working at height equipment prior to use. Report any defects immediately - Consider weather conditions – wet, windy and/or icy conditions can have a serious impact on safety at height - Beware of overhead obstructions, especially overhead power lines (metal ladders/metal reinforcements). When using ladders: - Always stand ladders on a firm base. - Never use rungs as a support for planks, or rest rungs on planks - Remove excessive mud, grease, etc. from footwear prior to climbing/descending a ladder - Always use both hands to climb/descend, and face the ladder, 3 points of contact - Do not carry loads up ladders – use hoists or alternatives - Never overreach from ladders – get down and move them. Roof work: - All roof work needs to be properly risk assessed before starting - If roof work is involved identify any fragile areas and/or openings and implement suitable protective precautions - Use crawling boards/roof ladders where applicable.